CTA Head and Neck 7/15/2025

|  | Siemens | GE |
| --- | --- | --- |
| Pre-contrast Brain | 3 x 3 mm recon Soft Tissue | 2.5 x 2.5 mm recon Standard |
| Head and Neck CTA | 1 x 1 mm recon Soft Tissue | 1.25 x 1.25 mm recon Standard |
| Post-contrast Brain | 3 x 3 mm recon Soft Tissue | 2.5 x 2.5 mm recon Standard |

| Head and Neck CTA post processing |  |
| --- | --- |
| For optimal image quality it is necessary to create a sub mm axial data set that is used to create reformats |  |
| 0.6 x .0.6 mm CTA axial recon for Siemens scanners 0.625 x 0.625 mm CTA axial recon for GE scanners |  |
| Oblique Axial thick MIP: from the base of the skull through the vertex of the head | 16 mm thick x 1mm increment |
| Coronal thin MIP: from the anterior skin surface of the head and neck through the posterior skin surface of the head and neck | 1mm thick x 1mm increment |
| Sagittal thin MIP from the left lateral skin surface of the head and neck through the right lateral skin surface of the head and neck | 1mm thick x 1mm increment |
| Coronal thick MIP: from the anterior skin surface of the head and neck through the posterior skin surface of the head and neck | 16 mm thick x 1mm increment |
| Sagittal thick MIP from the left lateral skin surface of the head and neck through the right lateral skin surface of the head and neck | 16 mm thick x 1mm increment |

PACS Series should be:

|  | Siemens | GE |
| --- | --- | --- |
| - Pre-contrast Brain | 3 x 3 mm recon Soft Tissue | 2.5 x 2.5 mm recon Standard |
| - Head and Neck CTA | 1 x 1 mm recon Soft Tissue | 1.25 x 1.25 mm recon Standard |
| - Thick MIP Oblique Axial | 16 mm thick x 1mm increment | 16 mm thick x 1mm increment |
| - Thin MIP Coronal | 1 x 1 mm recon Soft Tissue | 1.25 x 1.25 mm recon Standard |
| - Thin MIP Sagittal | 1 x 1 mm recon Soft Tissue | 1.25 x 1.25 mm recon Standard |
| - Thick MIP Coronal | 16 mm thick x 1mm increment | 16 mm thick x 1mm increment |
| - Thick MIP Sagittal | 16 mm thick x 1mm increment | 16 mm thick x 1mm increment |
| - Post-Contrast Brain | 3 x 3 mm recon Soft Tissue | - x 2.5 mm recon Soft Tissue |
| - 3D VR Rotation Head/Neck |  |  |

3D VRT Rotational Brain and Carotid from aortic arch with bone/soft tissue subtraction