Adult CTA Neck Trauma 01/11/2024

CTDI: ~10-25mGy

Setup:

- Neck positioned on the flat table top

- Retract shoulders as much as possible

- Head First Supine lateral scout from below the aortic arch through the bottom half of the orbits

DFOV: Preferred 15cm for CTA (18 cm can be used when the required anatomy is not displayed in a 15cm DFOV)

Preferred 12cm for Cervical Spine recontructions

Exam:

- CTA Neck

    - Scan from the floor of the sella through the level of arch

(includes great vessel orgins and most of the arch)

    - Smart Prep or Bolus Tracking in the descending aorta at level of arch

(trigger set at 80 HU on Siemens) (45 HU on GE)

    - 10 second monitoring delay on Bolus Tracking/Smart Prep

Contrast:

- at the discretion of the Radiologist

- 75ml of 320- 370 mg/dl non-ionic contrast @ 5 ml/sec with an 18 gauge catheter

- If unable to access an 18 gauge, please use a 20 gauge and adjust to 4ml/sec

Injector Setup:

- 30ml Saline Flush @ 5ml/sec

- 75ml Omnipaque 350 @ 5ml/sec

- 100ml Saline Flush @ 5ml/sec

PACS Series:

- 1x1 CTA  axial (source Images)

- 1x1 CTA Soft Tissue Coronal MIP

- 1x1 CTA Soft Tissue Sagittal MIP

Include the following Cervical Spine reconstructions from CTA scan when a CT Cervical Spine has also been ordered

Focused DFOV of 12cm from T1 through C1

- 1x1 Axial Cervical Soft Tissue

- 1x1 Axial Cervical Bone

- 1x1 Coronal/Sagittal